KPDY300H 24 Volt DC-DC, 300W ATX with P4 Connector

Top of Form SPECIFICATIONS. DC INPUT RATING; 24 Vdc 18 Vdc 36 Vdc 22 A(max.). OUTPUT RATING OUTPUT 1; OUTPUT 2; OUTPUT 3; OUTPUT 4; OUTPUT 5; OUTPUT 6. VOLTAGE; +5V; +3.3V; +12V; -5V; -12V; +5V SB. MAXIMUM LOAD; 35A; 14A; 12A; 1A; 2A; 1A. MINIMUM LOAD; 3A; 0A; 1A; 0A; 0A; 0A. PEAK; 38A; 21A; 16A; 3A; 2A. Total combined output of +3.3V and +5V shall not exceed 175W. RIPPLE / NOISE; 50mVp-p; 33mVp-p; p 100mVp-p; 50mVp-p; p 100mVp-p; 50mVp-p. LINE REGULATION; ±1%; ±1%; ±1%; ±1%; ±1%; ±1%. CROSS REGULATION; ±5%; ±5%; ±5%; ±2%; ±10%; ±5%.. Efficiency; 65% at typical load. Hold up time; 16ms at full load. Power good signal; 100~500ms. Shout Circuit Protection; A short circuit on any DC output will cause the power to latch.. Over power protection; Peak power protected to between 110% to 125% of continuous rated power.. Over voltage protection; +5V:5.5-6.5V. Teamperture range; 32°F ~122 °F ( 0~ 50°C). Relative Humidity; Operating: 10~90% rh Storage: 5%~90% rh. Safety Standards; Meets UL 1950, CSA 22.2 No.234, TUV IEC 950, FCC Class B, VDE Class B. M.T.B.F.; 100k hours at 25°C. Dimention; (W x H x D ) 150 x 86 x 142 mm/ 5 7/8"x3 3/8"x5 1/2".. ORDERING GUIDE. Model; Description; Price. KPDY300H; 24 Volt DC-DC, 300W ATX with P4 Connecto; $195.00.. Bottom of Form